- Chapter 4

- Stanislaw Jakubowski, Antonina Adamowicz - Hummel

##### The Use of Technical Devices in Setting Up Workplaces

for Visually Impaired Persons

- Blind and Low Vision Persons

Visual impairment occurs in varying degrees from small defects, which are easily compensated by appropriate adaptations such as optical and non-optical devices, to total blindness. The absence or presence of useful vision is the primary criterion in dividing the visually impaired into two groups: blind persons and persons with low vision.

Visual impairment may be a result of a disease of, or damage to, the visual system. Therefore among persons with low vision the ability to see varies greatly and depends on:

- the condition of the visual system and the extent of  residual vision,

- external factors, such as:

-	the type, intensity and location of lighting,

-	the size, shape and number of objects, their color and how they contrast with the environment,

-	exposure time, i.e. how much time the visually impaired person has to view an object, and the static nature of an image vs. its movement in space, and the

- physical and mental disposition of the visually impaired person.

When arranging workplaces for low vision persons it is important to ensure they have maximum opportunities for comfortably using their residual vision. This includes consideration of the appropriate location of light sources, providing ways for controlling its intensity, eliminating glare, providing rooms and objects with differentiated color contrast, and using low vision devices as required.

In addition, blind persons might also require a larger workplace as they may use more equipment, and may also need to store bulky Braille or large print materials.

However, key conditions for contributing to achievement and high productivity in the work environment are adequate skills, clear organization of work, adaptive techniques, and technical devices.

With blind persons, the technical devices used should, to the best extent possible, take the place of the missing vision. Persons with low vision frequently use the same kind of devices as blind persons. In addition, they have mastered the use of adaptive techniques and other optical or non-optical devices which contribute to enhancing their overall functional abilities.

Technical  devices for persons with low vision

The devices used by persons with low vision to improve their vision are generally divided into optical devices (a wide variety of lenses, prisms and combinations of these), optical-electronic devices (CCTV magnifiers and electronic magnifiers, also called electronic loupes); electronic devices (computer aided magnification) and non-optical devices, which are simple every day objects to facilitate the use of residual vision, or compensate for reduced vision.

# Optical devices

Unfortunately versatile low vision spectacles, which would improve vision in any circumstances, have not yet been designed. Therefore, visually impaired persons may need several kinds of optical devices, depending on the activity performed, for example, traveling, searching for objects, reading, writing, using a telephone and a fax machine, household chores, and watching TV. In addition, when fitting optical devices it is vital that each person be individually assessed.

These are the most common and useful optical devices:

- Telescopes - these devices provide magnification of distant objects. They come as monoculars or binoculars and may be hand-held or mounted in front of the eye (e.g., in a spectacle frame). They are very useful for stationary observation (e.g., of a construction site, a sports ground, a theater stage) and for spotting objects while traveling outdoors (e.g., street names and house numbers, numbers on buses, information signs, shop signs, etc.).

The most recent generation of telescopes, with a feature called “extra short focus,” allow for viewing from a very short distance, which in some cases can be as little as 30 centimeters. This feature, plus the option to fit a “reading cap” onto the telescope, can produce a more flexible device that allows the user to perform activities which require distant, middle range and near viewing.

A telescope with an extra short focus, mounted in a spectacle frame, is particularly useful for working with computer screens, as it leaves both hands free.

- Magnifiers are devices which provide magnification of near objects. They may be handheld, on a stand, mounted in spectacle frame, or fitted into lamps, etc. Some magnifiers have their own source of light, which is particularly useful when ambient lighting is insufficient, or in the case of high power magnifiers, where the very short focal length (the distance between the lens and the object viewed) does not allow for adequate illumination by ambient light.  (Fig. 8).

- Microscopes are magnifiers, usually of high power, mounted in a spectacle frame. They provide the user with a larger field of vision than other types of similar power magnifiers, and leave the hands free. (Fig. 8)

Well-known manufacturers of quality optical devices include: Eschenbach, COIL, Schweitzer and Zeiss in Europe; Specwell in Japan, and Walters in the USA.

## Fig. 8

### Magnifiers

Optical-electronic devices

This group of more sophisticated devices includes, first of all, the CCTV magnifier. It was developed in the USA in the early 1970s and is ideal for many individuals with low vision. The operating principle of a CCTV magnifier is the same as close circuit television. The object or text to be viewed rests on a movable table, and is first magnified by a system of lenses, then recorded by the camera and magnified again by internal electronic system. The image is then displayed on a screen which may be fairly large (17-20 inches). Some of these devices can provide magnifications up to a factor of 60. In addition, newer generation magnifiers offer a selection of color contrast between the object and its background which can be adjusted to suit each individual’s needs.

There is a variety of CCTV magnifiers, from small devices containing only the camera and the magnifying electronic system, which is connected to a standard TV monitor, to two-camera systems which enable users to work simultaneously with close objects (e.g., on a desk), and distant objects (e.g., on a whiteboard or a screen, or located in another part of the same room). There are also CCTV magnifiers that are computer compatible.

Well known European manufacturers of such equipment include: Tieman of the Netherlands, Reinecker of Germany and Magnilink of Sweden.

Another range of this type of equipment is electronic magnifiers, or loupes. The Liberty magnifier is among the most recent. This small, portable device is battery or mains powered. Its miniature cameras, which move on rollers, much like a computer “mouse”, may be easily manipulated for example across the page of a book, and the image is displayed on a screen with a maximum magnification of 9x. Better results can be achieved when the camera is wired to a computer screen; and the text or drawing can then be displayed at a magnification of up to 30x, depending on the size of the screen.

Magnifying images on a computer screen

There is increasingly larger evidence that text and graphics displayed on a computer screen can be read more easily by persons with low vision than the same information on paper, even if magnified. Such computer systems for text and graphics magnification may be divided into two groups: hardware- and software-generated.

The most typical arrangement in the hardware group is a CCTV magnifier coupled with a computer. A particularly interesting concept is offered by the Lynx system (the Telesensory company) in combination with a Vista VGA card, which results in a configuration that can serve two cameras and a video recorder. The image viewed may also be saved on a hard disk.

Some graphic cards, readily available in the computer market, can provide a 2x and 4x magnification around the cursor of a mouse. Best known are the Matrox cards. The magnification system is inferior to software arrangements, however, as it does not allow for changing the size of the areas selected; for example, it does not follow keyboard-generated changes. Magnified characters can also be achieved by the use of sufficiently large screens, e.g. 17, 20 or even 40 inches.

Word processors with character magnification options appeared on the market as early as the 1980s and were the forerunners of today’s more advanced software systems. Current development focuses mainly on graphic magnification systems operating under Windows. This class of programs includes, for example, Zoom Text by the American company Al Squared; Magic developed by another American company Joyce Hunter, and Lunar developed in the U.K. These programs can magnify highlighted text areas several dozen times, simulate a moving magnifying glass on the screen, and change image and background colors in an almost unlimited range. They are currently priced at 2,000 – 3,000 zlotys [$500-750].

# Non-optical devices

Non-optical devices, many of which effectively enhance residual vision, include but are not limited to:

- lighting and lighting control, such as: lamps, flashlights, blinds, dimmers, and sunglasses.

- image magnification without optical devices, including felt tip markers, bold line paper, large print material, large digit telephones (Fig. 9).

Fig. 9
Large digit telephones

- image stabilization and ensuring correct posture for visual tasks, including: adjustable portable reading stands, clipboards, typoscopes ("window" or stencil for reading, i.e., a rectangle of dark cardboard with a slot cut out which accommodates one or more lines of text and facilitates following the line of text while eliminating possible glare from the paper);

- highlighting certain elements of the environment to make them more visible, e.g., contrasting tape or paint to mark the edges of steps, light switches and power outlets, controls, dials, telephone buttons, etc.

These and many other devices for persons with low vision (described in more detail in Chapter 3) can significantly improve visual performance, and provide opportunities to function independently and with increased productivity in all aspects of everyday life.

Technical devices for blind persons

In the case of totally blind persons the most productive senses that remain are hearing and touch. Therefore devices designed for such individuals employ either or both these senses simultaneously. Initially, simple devices were developed, which were adapted to accommodate human tactile abilities.  More recent devices which rely on the sense of hearing are an outcome of technical developments.

Everyday life adaptations and devices

Early inventions for blind people are connected with the use of Braille (raised dot), which blind persons read with their fingertips. The system was invented in the first half of the 19th-century by a blind Frenchman, Louis Braille. His name has since been used not only in reference to the system of reading and writing used by blind persons but also for any kind of devices or appliances with markings in the form of raised dots. The basic equipment for writing Braille symbols are slate and stylus and are the equivalent of common tools used by sighted people, such as paper and pen or pencil. The rate of writing is significantly increased when a Braille typewriter is used. Other useful tactual devices include signature and writing guides (Fig. 10); measures and gauges with tactile markings, and watches with lids that open to reveal Braille that allows the individual to tactually “read” the clock face.

Fig. 10

###### Writing guides

Signature guides

Cassette recorders are among the most popular appliances used by blind persons. They are cheap and easy tools to record and reproduce voice information, available to both blind and sighted people. One major application which consists of recording literature on audio cassettes (often referred to as “talking books”) has become extremely popular worldwide. The Central Library of the Polish Association of the Blind has over 4,000 titles recorded in this format.

Portable voice notebooks, memo recorders and dictaphones are even better suited for the recording of short and frequently changed messages than cassette recorders as each message is a separate unit which may be erased safely, without damage to the neighboring messages. The latest models can be wired to a computer and messages can be saved on hard disc.

As far as activities of daily living are concerned, blind persons also use a variety of other auditory devices, e.g., liquid level indicators, talking scales, thermometers, etc.

In independent traveling, despite the emergence of such experimental devices as ultrasound obstacle detectors or the "electronic dog", blind persons still mainly use the white cane. What is important is the length of the cane; longer canes (reaching at least chest height) provide better protection for the traveler. In some countries, dogs of certain breeds are trained to become blind persons' guides, although in Poland, this resource has not yet been introduced on a significant scale.

One piece of equipment that cannot be overestimated as an aid to visually impaired persons is the mobile phone. Because of its constant technical readiness, it allows for instant communication with other people, facilitates speedy and independent travel, e.g., when a blind person traveling on a train orders a taxi to be waiting at the station, and, most importantly, enables rapid calls for assistance in case of emergency, etc.

Braille printers

The latest Braille printers have dramatically increased access to written information for blind persons. The devices are reasonably priced (from 10,000 PLN [$2,500]), and therefore can be used widely in schools, institutions for blind persons, and sometimes as privately owned equipment.

# Braille displays and notetakers

# Braille displays provide electro-mechanical output, consisting of elements called “modules,” each one showing a single Braille character, arranged in a linear sequence.

Depending on the number of modules, Braille displays may represent one line of the screen or a fragment. The configurations of Braille dots change automatically as the device continuously tracks text and graphics on the computer screen. The display  software can also interpret, in Braille symbols, various shapes of the cursor, character attributes and colors. Some Braille displays, often weighing no more than 2 kilos, can easily be detached from the computer and carried in a briefcase.

Smaller and even more portable are Braille notetakers. They have a seven-element Braille keyboard and a line of modules for reading Braille information. Typically, they are shorter than displays which require a solid connection to a computer and, dependent on type, may contain between one and forty elements. The internal memory in notetakers ranges between several and over a dozen megabytes, and can store large amounts of information including notes, handbooks, address databases, and even dictionaries. The data can be entered from the Braille keyboard or transferred from a computer. Conversion software allows printing of the text from internal memory of the notetaker to a Braille or conventional printer.  More recent notetakers, in addition to word processing functions, also include clocks/stopwatches, calendars, and calculators.

In many workplaces, Braille displays and notetakers are simply indispensable. Despite a high price (a device with a 40 character line costs around 25,000 PLN [$6,250]), these equipment should not be ignored when setting up workplaces for blind persons.   And, of course, as their production and utilization increases, current prices  will inevitably fall to more reasonable levels.

Speech output equipment

Blind persons receive most information through the sense of hearing, with speech being of particular importance. For that reason, concerted attempts to design effective talking machines have been made for over a decade. In English and German-speaking countries a whole range of talking devices for every day use is already available, such as calculators, thermometers, pressure gauges, compasses, scales, and watches/clocks (Fig. 11). In 1998 Polish-speaking watches were introduced to our market and more innovations and devices are expected.

## Fig. 11

###### Talking clock

It is only recently that blind persons have gained access to a color recognition device. Produced by the Austrian company CareTec, this small device, programmed to speak in English or German, is called ColorTest 150. It is battery powered and costs around 1,800 PLN [$50].

Further development of speech output has enabled the generation of artificial voice messages from the computer where both screen information and messages are keyed in by a blind user. Speech output, supplemented with special screen reading and keyboard interpreting software, has revolutionized information-retrieval by blind and visually impaired persons. As a result of these developments such individuals can use the computer efficiently -- and read and write independently.

At the moment, there are four types of speech synthesizers available in Poland:

1.	AlTalk by the Polish Altix company, with Readboard software, priced at 100 PLN [$120]. So far, this has the largest number of users in Poland.

2.	British Apollo synthesizer with Hal software, by the British Dolphin Systems company. It costs around 3,000 PLN [$750], and the Hal for Windows software, 2,000 PLN [$500]. As it is possible to install any 7 languages out of the 40 currently available, Apollo 2 is the most popular device in Europe.

3.	SMP speech synthesizer by the Polish company ECE, with dedicated software. It costs around 1,900 PLN [$480], including software.

4.	Polish synthesizer Kubuś by the Harpo company, has very high quality speech output. In the current absence of dedicated software, the English Hal program can easily be used. The price of the device is around 3,000 PLN [$750].

Key features of these speech output devices, such as: information transmission rates equal to the rate of speech; auditory control during typing, and relatively low prices, make them very popular among blind and low vision computer users.

Scanners

The emergence of scanners and print recognition software has been a major breakthrough for blind persons, and a combination of the two produces an effective reading device.

A scanner is a device that transforms traditional (analog) images into digital form and then to text in sequential characters, utilizing a recognition program known as OCR (Optical Character Recognition).

Blind people in Poland mainly use scanners by Hewlett-Packard and the Hungarian Recognita Reader 3.0 software, which has been developed especially for blind persons. Windows users have an even better program, FineReader 5.0 [author's update 2001].

For good print quality, accuracy of the character recognition may exceed 99 percent. The text is automatically saved as a disc file and may be read by a blind person using a speech output device, a Braille display, character magnification software, or a Braille printer.

The latest devices combine the three key processes of scanning, printed characters recognition and speech output into one process that reads text aloud. There are already several designs for this type of device, worldwide. The first was the Kurzweil Talking Machine, built by the American scientist Raymond Kurzweil in 1980.  Polish engineers have also designed a reading device, combining a computer, scanner with character recognition software, and speech output. The device is marketed by the Poznań-based company Harpo under the name Autolektor, and is priced at around 17,000 PLN [$4,200].  Its simple Braille marked keyboard and voice instructions allow blind persons with practically no computer skills to use it effectively.

Windows access

The vast majority of computer users around the world have adopted the Windows graphic based operating system. Windows 98 in particular is a convenient information technology (IT) tool for persons with low vision; it offers magnification of all objects and edited texts. Such persons find it much easier to locate the differently shaped icons on the screen than to struggle with the text complexities of DOS commands. However, totally blind individuals, unless they use special software, are unable to utilize Windows, as its operating system is not compatible with speech output devices and Braille displays equipped with traditional DOS software.

However almost simultaneously with the worldwide spread of Windows systems, several companies in the USA started work on software to provide a bridge between graphic-based systems and blind users.

As early as 1989 the first screen reading package appeared, called OutSpoken, by Berkeley Systems, Inc. for computers with graphic user interface. The same company has also developed versions of OutSpoken for MS Windows 95 and 98.

One of the key concepts is the "smart screen" developed by Henter-Joyce. In its new version, 3.1 and known as JAWS, (Job Access With Speech). The company has introduced a number of modules, which constantly follow screen changes; the changes are analyzed and evaluated, and then the most significant ones are announced to the user.

Currently, the OutSpoken and the Jaws are among the best systems. Others include Windows Eyes and Hal for Windows. All these programs support a range of speech output systems, and some support most types of Braille displays. All of them have also been adapted to the Polish language.

Instead of a mouse, these programs use a keyboard, often with unique key combinations. The software cost between 1,800 PLN [$450] (Hal for Windows) and 3,600 PLN [$900] (JAWS).

Applications of electronic devices in workplaces created for blind and low vision persons

It is obviously evident that information technology is particularly useful for persons with visual impairments. A key component in all these arrangements is, of course, a computer.  Currently, as discussed above, the most important adaptive devices include:

- Braille displays,

- software for writing Braille using the computer keyboard,

- voice and Braille notetakers,

- Braille printers,

- voice output devices for reading print.

Information technology has also brought about other significant improvements for persons with low vision, including:

- CCTV magnifiers,

- electronic magnifiers,

- software for magnification of characters on a computer screen,

- word processors with larger characters.

As a result of these inventions, visually impaired persons have obtained access to most IT developments, including, in particular: word processors; electronic dictionaries; scanners with character recognition software; printers; sound cards and modems.

Modern technology has indeed opened up vast new possibilities for effective communication and work productivity to persons with vision impairments, by including:

a) 	independent access to written information via scanners, data carriers or modems,

b) 	independent preparation of texts on a computer,

c)	output in the form of printouts or on digital carriers, immediately accessible to others.

Once information is available on a digital carrier, it can be reproduced without any difficulty by a computer with speech output or a Braille display. Persons with low vision may use any of the character magnification software options.

Using a printer and generating legible texts in any form is no longer a problem for blind persons or those with low vision.  IT has, and is rapidly continuing to, help solve the problem of communication for blind persons with regard to text information, which until recently was the main obstacle in employing them to work alongside sighted people.

Examples of blind persons working among sighted people

Wanda lost her vision in infancy. From secondary school onwards, she was educated among sighted persons. Linguistic studies prepared her for work as translator/interpreter and language teacher. She married and now has four adult children. She has always been able to combine being a mother with her job, and has never been short of work. Currently, she is employed in a ministry. Apart from typical office tasks (writing memos, preparing correspondence, giving information over the telephone, etc.) she edits brochures and undertakes a large amount of written translation and oral interpretation.

Working with the computer she uses an eighty character Braille display, a scanner with Recognita software for character recognition, an inkjet and Braille printers, and a portable dictaphone. She commutes to work on a city bus, traveling with a white cane.

Owing to her excellent skills she is constantly offered additional employment opportunities. She now translates texts in economics and statistics and other disciplines, including plant genetics; interprets at scientific conferences, and adapts handbooks and music scores for Braille use.  In addition, she is vice-president of an NGO.

Stefan has low vision. He works as an IT engineer. Currently he is administrator of a large central database in an insurance company. His duties include monitoring the efficient operation of the system, collecting and archiving data from all over Poland, writing periodic reports on the basis of the data in the database, and programming.

His low vision is largely compensated for by the use of a speech synthesizer (Polish and English), character magnification software, and a scanner with character recognition software. Sometimes he uses a regular magnifier.

His high competence in IT has gained him wide recognition as an excellent teacher of computer technology to blind persons. Recently Stefan has been very successful teaching blind persons to use Windows, and he is now approached with offers to run courses all over Poland.

These two examples certainly do not exhaust all the types of work that are currently being conducted by visually impaired persons. It is also impossible to design a workplace that would suit everybody, because - as it has been said before in this guidebook - each person and each type of work requires customized and individual technical devices and other adaptations.

The high price of some technical devices may be an initial concern. For example, CCTV magnifiers may cost between 1,500 [$370] and 15,000 PLN [$3,700], and the price of an 80 character Braille display may be as much as 50,000 PLN [$12,500]. However, the cost of adapting a workplace to the needs of a visually impaired employee and the provision of all necessary equipment, may be reimbursed, in full or in part, by the State Fund for the Rehabilitation of Persons with Disabilities (PFRON).  This benefit is offered to employers by current legislation and regulations and greatly increases the feasibility and desirability of employing visually impaired individuals in a wide range of roles and companies throughout Poland. (These issues are discussed in the next section. See also Appendix 10.)